# STANDARD OPERATING PROCEDURE

## Cyclohexane

*Delete items in red after you have added all your lab-specific information. Modify general information as needed.

| 1. LAB INFORMATION | Building: Room(s): Department: Principal Investigator Name: Procedure: |
| --- | --- |
| 2. PHYSICAL & HEALTH HAZARDS | Cyclohexane- CAS# 110-82-7; also known as Benzene hexahydride; Hexahydrobenzene; and Hexamethylene is a colorless liquid with a sweet, pungent odor used as a solvent and paint remover, in analytical chemistry, and in making adhesives and perfumes. - Highly flammable liquid and vapor. May form explosive mixtures in air. - Causes skin irritation. - Causes serious eye irritation. - May cause drowsiness or dizziness. - May be fatal if swallowed and enters airways. - May damage the liver and kidneys. GHS hazard pictograms Signal Word: DANGER Exposure Limits: DOSH: TWA: 300 ppm; STEL: 375 ppm NIOSH: TWA: 300 ppm ACGIH: TLV: 100 ppm Toxicological Data: ORAL (LD50): 6,240 mg/kg [Rat] DERMAL (LD50): >2,000 mg/kg [Rabbit] INHALATION (LC50): 19.07 mg/l 4 hours [Rat – Male and Female] *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. TRAINER / RESOURCE PERSONNEL | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for Cyclohexane must be available at all times to all personnel working in the laboratory. The SDS used in this laboratory are located in Building, Room and/or Electronic Location. Labeling: Cyclohexane containers shall either have original manufacturer’s label affixed or a self-made workplace label identifying the contents and hazards of the chemical at the minimum. Chemical abbreviations and formulae should be avoided unless a legend is present in the lab. For information on workplace labels, see https://ehs.wsu.edu/ohs-chemhazardcommunication/ohs-workplacelabels/. |
| 5. PROTECTIVE EQUIPMENT | Wear nitrile, neoprene, butyl, or Viton gloves, chemical splash goggles, and a fully buttoned lab coat at the minimum. ANSI approved safety glasses may be appropriate when properly working behind a fume hood’s protective sash. When working with high hazard flammable liquids with an increased risk of splash or spill exposure (e.g. transfers from a larger container), consider siphon transfer for improved control or additional PPE, such as thick-cut butyl rubber or equivalent gloves, face shield and a flame resistant lab coat. Wash hands immediately after handling flammable liquids and removing gloves. - Note: Always consult with your glove manufacturer’s glove compatibility chart to ensure selected gloves are compatible with specific chemical being used. Full length pants and close-toed shoes are required to be worn at all times. Insert lab specific information on personal protective equipment required for use. Always work within a properly functioning, face velocity certified laboratory chemical fume hood. Insert lab specific information on ventilation controls and equipment used to control exposure. |
| - WASTE DISPOSAL PROCEDURES | Waste Cyclohexane must be managed as Dangerous Waste. Collect solution in a compatible container (e.g. a polyethylene container provided by EH&S). Specialized waste containers and/or lids may be required for comingled wastes where reactions may generate heat or pressure. Waste containers should be stored away from incompatible materials (reference Section 10 of the SDS for incompatible materials). A completed Dangerous Waste label must be attached to dangerous waste containers before waste is added. When the container is full or no longer being used, submit an online Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator (if applicable) at Building, Room, Phone Number. Dangerous waste information and instructions can be found at the following link: https://ehs.wsu.edu/Chemical-Waste/ Do not dispose of Cyclohexane waste down the drain or in the regular waste stream. For assistance evaluating dangerous waste characteristics, contact EH&S https://ehs.wsu.edu/contact-information/. Insert lab specific information on Cyclohexane waste collection procedures, location, storage and handling. |
| 7. DESIGNATED AREA INFORMATION | The Cyclohexane is stored and dispensed in Building, Room. Confine all work with Cyclohexane to a properly functioning certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in Laboratory’s Chemical Hygiene Plan (CHP). Insert lab specific information on Cyclohexane storage and use locations. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes and seek medical attention. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and seek medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Release: If a significant amount of Cyclohexane is spilled outside the fume hood, immediately evacuate, secure area, notify others in the vicinity, and call 911 if there is an immediate threat of fire or the spill is uncontained. If personnel are exposed and need medical attention, call 911. If the spill is contained, secure the area, evacuate and call EH&S at 509-335-9000. Small Release: If a small amount of Cyclohexane is spilled (it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel shall wear nitrile, neoprene, butyl, or Viton gloves, chemical splash goggles, and a fully-buttoned lab coat. Absorb with an inert dry material and place material in an appropriate waste disposal container (resealable bag, etc.) and dispose of as hazardous waste (see above WASTE DISPOSAL PROCEDURES). Additional PPE such as respirators may be necessary depending upon material and concentration. You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator. If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and follow the large release instructions above. As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on Cyclohexane can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). Insert lab specific information on Cyclohexane spill cleanup procedures and applicable location specific emergency procedures. |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | - Store Cyclohexane containers upright in a designated, labeled area such as a chemical storage cabinet. Secondary containment such as a Nalgene/polypropylene tub is recommended. Cyclohexane containers should always be stored below eye level. Storage area should be a secured, cool and well-ventilated area away from direct sunlight, heat, sparks, flame, or other sources of ignition. - Store Cyclohexane in a designated, labeled, secure storage area away from other types of chemicals. Avoid incompatible chemicals such as strong oxidizing agents, nitrates, nitrogen dioxide, and dinitrogen tetraoxide (refer to Section 10 of SDS). If storage space is limited, use secondary containment such as a Nalgene/polypropylene tubs. - Store in tightly closed original container until ready for use. Immediately close all containers after use. Containers should be in good condition and compatible with material. - No more than 10 gallons (37 liters) of flammable and combustible liquid is permitted to be stored outside of a designated flammable liquid storage area (e.g. flammable storage cabinet). - Avoid using ignition sources (e.g. flame burners, hot plates, electrical equipment with damaged wiring, etc.) and/or creating static electricity in areas where flammable/combustible liquids are being used. - Ground and bond containers when transferring more than 4 liters of flammable or combustible liquids. - Transport Cyclohexane between locations in secondary containment, such as polyethylene or other non-reactive, non-breakable bottle carrier. - Purchase and store smallest quantities and lowest concentration needed for experimentation and what can be stored safely in the laboratory. - Personnel must store flammable and combustible liquids in accordance with the International Fire Code (IFC). Contact EH&S with questions (509-335-3041). Consult SPPM 8.12 Flammable/Combustible Liquid Storage for more info on WSU policy of flammable and combustible liquid storage. Insert additional lab specific information on Cyclohexane controls. |

- Certificate of Employee Training

The Principal Investigator must certify that they have provided training to lab personnel on the content of this SOP before work with Cyclohexane is performed.

| PI Name | PI Signature | Date |
| --- | --- | --- |

The following individuals attest that they have received training from the Principal Investigator on the content of this SOP.

| Name | Signature | Date |
| --- | --- | --- |