# STANDARD OPERATING PROCEDURE

## Hydroiodic Acid

*Delete items in red after you have added all your lab-specific information. Modify general information as needed.

| 1. LAB INFORMATION | Building: Room(s): Department: Principal Investigator Name: Procedure: REMEMBER! Always add acid to water to limit exothermic reactions such as splatter, bubbling and fuming. A large amount of heat is generated when strong acids are mixed with water. Adding more acid releases more heat. Adding water to acid forms an extremely concentrated solution of acid. Heat causes the solution to boil violently, splashing concentrated acid out of the container. Adding acid to water, the resulting solution is dilute and the heat released is absorbed by the water. Always Add Acid to water. |
| --- | --- |
| 2. PHYSICAL & HEALTH HAZARDS | Hydroiodic Acid- CAS# 10034-85-2; also known as Iodane, hydriodic acid, hydrogen iodide and hydronium iodide is a colorless to pale yellow liquid. - Corrosive chemical. May severely irritate and burn the skin and eyes with serious eye damage. May be absorbed through the skin. - Corrosive to metals and to tissue. - May cause respiratory irritation of the nose, throat, and lungs. - Long-term inhalation of low concentrations or short-term inhalation of high concentrations may result in adverse health effects. GHS hazard pictograms Signal Word: DANGER Exposure Limits: - No occupational exposure limits have been established for Hydroiodic acid. However, it may pose a health risk. Always follow safe work practices. *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. TRAINER / RESOURCE PERSONNEL | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for Hydroiodic acid must be available at all times to all personnel working in the laboratory. The SDS used in this laboratory are located in Building, Room and/or Electronic Location. Labeling: Hydroiodic acid containers shall either have original manufacturer’s label affixed or a self-made workplace label identifying the contents and hazards of the chemical at the minimum. Chemical abbreviations and formulae should be avoided unless a legend is present in the lab. For information on workplace labels, see https://ehs.wsu.edu/ohs-chemhazardcommunication/ohs-workplacelabels/. |
| 5. PROTECTIVE EQUIPMENT | Wear nitrile, neoprene, butyl, or Viton gloves, chemical splash goggles, and a fully buttoned lab coat at the minimum. Double gloving is recommended. ANSI approved safety glasses may be appropriate when properly working behind a fume hood’s protective sash. Always wash hands after removing gloves. - Note: Always consult with your glove manufacturer’s glove compatibility chart to ensure selected gloves are compatible with specific chemical being used. Full length pants and close-toed shoes are required to be worn at all times. Insert lab specific information on personal protective equipment required for use. Always work within a properly functioning, face velocity certified laboratory chemical fume hood. Insert lab specific information on ventilation controls and equipment used to control exposure. |
| - WASTE DISPOSAL PROCEDURES | Waste Hydroiodic Acid must be managed as Dangerous Waste if the solution has a pH of 6 or lower. Collect solution in a compatible container (e.g. a polyethylene container provided by EH&S). Specialized waste containers and/or lids may be required for comingled wastes where reactions may generate heat or pressure. Waste containers should be stored away from incompatible materials (reference Section 10 of the SDS for incompatible materials). A completed Dangerous Waste label must be attached to dangerous waste containers before waste is added. When the container is full or no longer being used, submit an online Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator (if applicable) at Building, Room, Phone Number. Dangerous waste information and instructions can be found at the following link: https://ehs.wsu.edu/Chemical-Waste/ If the solution has a pH between 6 and 9 and does not meet the definition of toxic in WAC 173-303-100 or exhibit other dangerous waste characteristics, it MAY be able to be drain discharged/disposed. For assistance evaluating dangerous waste characteristics, contact EH&S https://ehs.wsu.edu/contact-information/. Do not neutralize dangerous wastes for disposal, this may only be done at permitted treatment and disposal facilities. Insert lab specific information on Hydroiodic acid waste collection procedures, location, storage and handling. |
| 7. DESIGNATED AREA INFORMATION | The Hydroiodic acid is stored and dispensed in Building, Room. Diluted acid solutions using this chemical are prepared immediately prior to use in Building, Room. Confine all work with Hydroiodic acid to a properly functioning certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in Laboratory’s Chemical Hygiene Plan (CHP). Insert lab specific information on Hydroiodic acid storage and use locations. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes and seek medical attention. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and seek medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Release: If a significant amount of Hydroiodic acid is spilled outside the fume hood, immediately evacuate, secure area, notify others in the vicinity, and call 911 if there is an immediate threat of fire or the spill is uncontained. If personnel are exposed and need medical attention, call 911. If the spill is contained, secure the area, evacuate and call EH&S at 509-335-9000. Small Release: If a small amount of Hydroiodic acid is spilled, (e.g. it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel should wear at minimum, nitrile, neoprene, butyl or Viton gloves, chemical splash goggles, and a fully-buttoned lab coat. Absorb with an inert dry material and place material in an appropriate waste disposal container (resealable bag, etc.) and dispose of as hazardous waste (see above WASTE DISPOSAL PROCEDURES). Please do not use a neutralizer to clean up spill unless you are currently in the respiratory protection program and have been properly trained. Additional PPE such as respirators may be necessary depending upon material and concentration. You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator. If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and follow the large release instructions above. As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on Hydroiodic acid can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). Insert lab specific information on Hydroiodic acid spill cleanup procedures and applicable location specific emergency procedures. |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | - Store Hydroiodic acid containers upright in a designated, labeled area such as a chemical storage cabinet. Secondary containment such as a Nalgene/polypropylene tub is recommended. Hydroiodic acid solutions should always be stored below eye level. Storage area should be a secured, cool and well-ventilated area away from direct sunlight, heat, sparks, flame, or other sources of ignition. - Store Hydroiodic acid in a designated, labeled, secure storage area away from other types of chemicals. Avoid incompatible chemicals such as metals, water, organic materials, alcohol, magnesium, potassium and perchloric acid (refer to Section 10 of SDS). If storage space is limited, use secondary containment such as a Nalgene/polypropylene tubs. - Store in tightly closed original container until ready for use. Immediately close all containers after use. Containers should be in good condition and compatible with material. - Transport hydroiodic acid between locations in secondary containment, such as polyethylene or other non-reactive, non-breakable bottle carrier. - Purchase and store smallest quantities and lowest concentration needed for experimentation and what can be stored safely in the laboratory. Insert additional lab specific information on Hydroiodic acid controls. |

- Certificate of Employee Training

The Principal Investigator must certify that they have provided training to lab personnel on the content of this SOP before work with Hydroiodic acid is performed.

| PI Name | PI Signature | Date |
| --- | --- | --- |

The following individuals attest that they have received training from the Principal Investigator on the content of this SOP.

| Name | Signature | Date |
| --- | --- | --- |