# STANDARD OPERATING PROCEDURES FOR HAZARDOUS AND PARTICULARLY HAZARDOUS CHEMICALS

For

## Imidazole

| 1. PROCEDURE / PROCESS | Imidazole is used in Building, Room. Insert procedure here: |
| --- | --- |
| 2. CHEMICAL NAME(S) and associated PHYSICAL and HEALTH HAZARDS | Imidazole – CAS# 288-32-4; also known as 1H-imidazole, 1,3-diaza-2,4-cyclopentadiene, 1,3-diazole, and glyoxaline, is a white to colorless solid that is soluble in water producing a mildly alkaline solution with an amine-like odor. - Corrosive material. Causes burns by all exposure routes. - Imidazole causes severe skin burns and eye damage. - May damage the unborn child. - Harmful if swallowed. - May form combustible dust concentrations in air. Signal Word: DANGER Exposure Limits: No occupational exposure limits have been established for this chemical. This does not mean that this substance is not harmful. Safe work practices should always be followed. Toxicological Data: ORAL (LD50): 970 mg/kg [Rat] *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. NAME OF TRAINER / RESOURCE PERSON | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for Imidazole is located in the Laboratory Safety Manual located in Building, Room. Labeling: In addition to the standard label that identifies contents, hazards, precautionary measures, and emergency contact information, containers should also be marked as corrosive and reproductive toxin. |
| 5. PROTECTIVE EQUIPMENT | Wear chemical safety goggles, nitrile gloves and a fully-buttoned lab coat. (Note: Always check the manufacturer’s glove compatibility chart for proper glove selection.) Wash hands after removing gloves. Always work within a properly functioning certified laboratory chemical fume hood. An emergency eyewash unit must be within fifty feet and ten seconds of chemical usage area when working with this substance. An emergency shower may also be necessary if potential exists for large areas of the body to be exposed to this substance. |
| - WASTE DISPOSAL PROCEDURES | Waste Imidazole must be collected in its pure form and solutions. It should be collected in a sealable, airtight, compatible waste container. The container should be stored away from incompatible materials such as strong oxidizers, acids, acid anhydrides, and acid chlorides. A completed Dangerous Waste label should be attached when waste is first added to the container. When container is full or no longer being used, complete a Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator at Building, Room, Phone Number. |
| 7. DESIGNATED AREA INFORMATION | The imidazole is stored and dispensed in Building, Room. Always work in a properly functioning, certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in Laboratories Chemical Hygiene Plan. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes and seek medical attention. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and immediately seek medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Spill: If a large amount of Imidazole is spilled outside the fume hood, immediately evacuate and secure area and call 911 to contact EH&S. Small Spill: If a small amount of Imidazole is spilled (it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel should wear at the minimum chemical safety goggles, nitrile gloves and a fully-buttoned lab coat. Additional PPE such as respirators may be necessary depending upon material and concentration. (Note: You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator). If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and call 911 to contact EH&S. Use appropriate tools to clean up the spilled material. Clean up the area and place material in an appropriate waste disposal container (resealable bag, etc.) and dispose of as hazardous waste (see above WASTE DISPOSAL PROCEDURES). As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on Imidazole can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | Store in a tightly closed container in a secured, cool, dry and well-ventilated area. Store as a corrosive material. Store segregated from incompatible chemicals (below). Store away from direct sunlight, heat, sparks, flame or any other source of ignition. Avoid dust formation. May form explosive concentrations in air. Keep away from incompatible chemicals such as strong oxidizers, acids, acid anhydrides, and acid chlorides. |

Certification of Hazard Assessment

Is this document a certification of Hazard Assessment for the processes identified within?                            			Yes					No

If yes, provide the name of the person certifying the Hazard Assessment and the date it was performed:

____________________________________________________________________________________

Name									Date

The location of the Hazard Assessment is indicated in the document preceding this form.

Certificate of Employee Training

Name of person providing training for employees working with this process:

___________________________________________________________________________________

The following employees have been trained in when, where and how to use selected PPE, the maintenance, limitations and disposal of the PPE selected, and have demonstrated the correct use of the PPE selected on the reverse of this certification.

Name								Date Trained

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________