# STANDARD OPERATING PROCEDURE

## Inorganic Acids

*Delete items in red after you have added all your lab-specific information. Modify general information as needed.

| 1. LAB INFORMATION | Building: Room(s): Department: Principal Investigator Name: Examples of Inorganic Acids covered by this SOP: Boric acid; Hydrobromic acid; Hydrochloric acid. Iodic acid; Hydroiodic acid; Meta-Phosphoric acid. Periodic acid; Phosphoric acid; Phosphorous acid. Silicic acid; Sulfamic acid; Sulfuric acid. Sulfurous acid; Tetrafluoroboric acid; Nitric acid*. *Note: Nitric acid is also a strong oxidizer. This SOP supports the use of inorganic acids. Inorganic acids derive from inorganic compounds. For the purposes of this SOP, all inorganic acids form hydrogen ions and an anion or conjugate base ions when dissolved in water. Common inorganic acids include hydrochloric acid and sulfuric acid. |
| --- | --- |
| 2. PHYSICAL & HEALTH HAZARDS | Inorganic acids can be harmful when ingested, absorbed through the skin, or when vapors are inhaled. They can cause skin burns, irritation, and eye damage, potentially leading to blindness. Ingestion can cause severe injury to the digestive tract leading to death. Inhalation can irritate and damage the respiratory tract including nose, throat and lungs and cause burning pain in the nose and throat, coughing, wheezing, shortness of breath and pulmonary edema. Fumes can be suffocating. Inorganic acids may have additional hazards associated with them, such as water reactive, oxidizer, or toxic. Add or delete GHS hazard pictograms accordingly. *Always refer to the Safety Data Sheet for the most detailed information* - Note: If working with Hydroflouric acid (not a strong acid) or other acids with acute systemic toxicity with skin exposure (e.g. hexafluorophosphoric acid), their respective chemical specific SOPs shall be used. Perchloric acid use shall reference a chemical specific SOP, addressing the potential formation of shock sensitive organic perchlorates. REMEMBER! When preparing aqueous solutions, always add inorganic acids to water to limit exothermic reactions such as splatter, bubbling and fuming. A large amount of heat is generated when strong acids are mixed with water. Adding more acid releases more heat. Adding water to acid forms an extremely concentrated solution of acid. Heat causes the solution to boil violently, splashing concentrated acid out of the container. Adding acid to water, the resulting solution is dilute and the heat released is absorbed by the water. When preparing aqueous acid solutions, Always Add Acid to water. |
| 3. TRAINER / RESOURCE PERSONNEL | Principal Investigator must guarantee all laboratory personnel receive training on the content of this SOP. Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | Safety Data Sheets (SDS) for inorganic acids must be available at all times to all personnel working in the laboratory. The SDS used in this laboratory are located in Building, Room and/or Electronic Location. Labeling: Inorganic acid containers shall either have original manufacturer’s label affixed or a self-made workplace label identifying the contents and hazards of the chemical at the minimum. Chemical abbreviations and formulae should be avoided unless a legend is present in the lab. For information on workplace labels, see https://ehs.wsu.edu/ohs-chemhazardcommunication/ohs-workplacelabels/. |
| - PROTECTIVE EQUIPMENT | Wear nitrile rubber gloves, ANSI approved chemical splash goggles, and a fully buttoned lab coat at minimum. ANSI approved safety glasses may be appropriate for lower hazard inorganic acids or when properly working behind a fume hood’s protective sash. When working with concentrated inorganic acids with an increased risk of splash or spill exposure (e.g. transfers from a larger container), consider siphon transfer for improved control or additional PPE, such as thick-cut neoprene, butyl rubber, Viton or equivalent gloves, a face shield and a chemical-resistant apron. Wash hands immediately after handling acids and removing gloves. - Note: Always consult with your glove manufacturer’s glove compatibility chart to ensure selected gloves are compatible with specific chemical being used. Full length pants and close-toed shoes are required to be worn at all times. Insert lab specific information on personal protective equipment required for use. Always work with inorganic acids within a properly functioning, face velocity certified laboratory chemical fume hood. Insert lab specific information on ventilation controls and equipment used to control exposure. If inorganic acids are used outside of a chemical fume hood, respiratory protection may be required which require affected personnel to be medically cleared, fit tested and enrolled in WSU’s Respiratory Protection Program (RPP). |
| - WASTE DISPOSAL PROCEDURES | Waste Inorganic Acid must be managed as a Dangerous Waste if it has a pH of 6 or lower. Inorganic acid wastes should be collected in a compatible container (e.g. a polyethylene container provided by EH&S). Specialized waste containers and/or lids may be required for comingled wastes where reactions may generate heat or pressure. Waste containers should be stored away from incompatible materials (such as bases and organic acids). Reference Section 10 of the SDS for incompatible materials. A completed Dangerous Waste label must be attached to dangerous waste containers before waste is added. When the container is full or no longer being used, submit an online Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator (if applicable) at Building, Room, Phone Number. Dangerous waste information and instructions can be found at the following link: https://ehs.wsu.edu/Chemical-Waste/ If the solution has a pH between 6 and 9 and does not meet the definition of toxic in WAC 173-303-100 or exhibit other dangerous waste characteristics, it MAY be able to be drain discharged/disposed. For assistance evaluating dangerous waste characteristics, contact EH&S https://ehs.wsu.edu/contact-information/. Do not neutralize dangerous wastes for disposal, this may only be done at permitted treatment and disposal facilities. Insert lab specific information on inorganic acid waste collection procedures, location, storage and handling. |
| - DESIGNATED AREA INFORMATION | Inorganic acids are stored and dispensed in Building, Room. Diluted inorganic acid solutions are prepared immediately prior to use in Building, Room. Confine all work with strong inorganic acid to a properly functioning certified laboratory chemical fume hood. The designated storage and use area(s) should be shown on the floor plan in your laboratory’s Chemical Hygiene Plan. Insert lab specific information on inorganic acid storage and use locations. |
| - DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes and seek medical attention. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and seek medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Release: If a significant amount of inorganic acid is spilled outside the fume hood, immediately evacuate, secure area, notify others in the vicinity, and call 911 if there is an immediate threat of fire or the spill is uncontained. If personnel are exposed and need medical attention, call 911. If the spill is contained, secure the area, evacuate and call EH&S at 509-335-3041. Small Release: If a small amount of inorganic acid is spilled (it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel shall wear nitrile, neoprene, or butyl gloves, chemical splash goggles, and a fully-buttoned lab coat. Sweep up carefully and avoid generating dust (solids) or absorb with an inert dry material (liquids) and place material in an appropriate waste disposal container (resealable bag, etc.) and dispose of as dangerous waste (see Section 6: WASTE DISPOSAL PROCEDURES). Please do not use a neutralizer to clean up spill unless you are currently in the respiratory protection program and have been properly trained. Additional PPE such as respirators may be necessary depending upon material and concentration. You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator. If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and follow the large release instructions above. Insert lab specific information on inorganic acid spill cleanup procedures and applicable location specific emergency procedures. |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | - Store acid containers upright in a designated area such as a corrosive storage cabinet or a storage cabinet under fume hood designed for chemical storage. Storage area should be a secured, cool and well-ventilated area away from direct sunlight, heat, sparks, flame, or other sources of ignition. - Store in tightly closed original container until ready for use. Containers should be in good condition and compatible with material. Do not store in metal containers. - Store segregated from incompatible chemicals such as bases and organic acids (refer to Section 10 of SDS). - Organic acids, inorganic acids, and oxidizing acids must be stored separately or in secondary containment such as a Nalgene/polypropylene tub. - Transport all inorganic acids in secondary containment, such as polyethylene or other non-reactive and non-breakable acid/solvent bottle carrier. Insert additional lab specific information on inorganic acid controls. |

- Certificate of Employee Training

The Principal Investigator must certify that they have provided training to lab personnel on the content of this SOP before work with Inorganic acids is performed.

| PI Name | PI Signature | Date |
| --- | --- | --- |

The following individuals attest that they have received training from the Principal Investigator on the content of this SOP.

| Name | Signature | Date |
| --- | --- | --- |