# STANDARD OPERATING PROCEDURES FOR HAZARDOUS AND PARTICULARLY HAZARDOUS CHEMICALS

For

Paraformaldehyde

| 1. PROCEDURE / PROCESS | Paraformaldehyde is used in Building, Room. Insert procedure here: |
| --- | --- |
| 2. CHEMICAL NAME(S) and associated PHYSICAL and HEALTH HAZARDS | Paraformaldehyde CAS# 30525-89-4 Also known as Metaformaldehyde, Aldacide, Flo-Mor, Formagene, Paraform; Polyoxymethylene; Triformol; Trioxymethylene. - Flammable Solid. Poisonous gases are produced in fire, including formaldehyde. - Harmful if swallowed or inhaled. Repeated inhalation of dust can produce varying degree of respiratory irritation or lung damage. - May be absorbed through the skin. Causes skin irritation. May cause an allergic skin reaction. Repeated skin exposure can produce local skin destruction, or dermatitis. - Causes serious eye damage. Repeated exposure of the eyes to a low level of dust can produce eye irritation. - May cause respiratory irritation. - Suspected of causing cancer. - Repeated or prolonged exposure to the substance can produce target organs damage, especially to the kidneys. Signal Word: DANGER Exposure Limits: DOSH: TWA: 0.75 ppm; STEL: 2.0 ppm Toxicological Data: ORAL (LD50): 800 mg/kg [Rat]. DERMAL (LD50): 10,000 mg/kg [Rat]. INHALATION (LC50): 1,070 mg/m 4 hours [Rat]. *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. NAME OF TRAINER / RESOURCE PERSON | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for Paraformaldehyde is located in the Laboratory Safety Manual in Building, Room. Labeling: In addition to the standard label that identifies contents, hazards, precautionary measures, and emergency contact information, containers should also be marked as Cancer Hazard. |
| 5. PROTECTIVE EQUIPMENT | Wear chemical safety goggles, nitrile gloves, and fully-buttoned lab coat. Other suitable gloves include butyl and Viton. A face shield may be worn to supplement safety goggles, but a face shield should NOT be worn in place of chemical safety goggles. Wash hands after removing gloves. Work within a properly functioning, certified laboratory chemical fume hood. |
| - WASTE DISPOSAL PROCEDURES | Waste Paraformaldehyde should be collected in a sealable, airtight, compatible waste container. The container should be stored away from incompatible materials such as oxidizing agents, reducing agents, metals, and acids. A completed Dangerous Waste label should be attached when waste is first added to the container. When container is full or no longer being used complete a Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator at Building, Room, Phone Number. |
| 7. DESIGNATED AREA INFORMATION | The paraformaldehyde is stored and dispensed in Building, Room. Always work within the confines of a properly functioning, certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in the laboratory’s Chemical Hygiene Plan. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes. Seek medical attention. For skin contact over a large portion of the body, remove contaminated clothing and shoes then wash with copious amounts of water at an emergency shower station for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and seek immediate medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Spill: If a large amount of Paraformaldehyde is spilled, immediately evacuate, secure area and call 911 to contact EH&S. Small Spill: If a small amount of Paraformaldehyde is spilled (it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel should wear at the minimum nitrile, butyl, or Viton gloves, chemical splash goggles, and a fully-buttoned lab coat. Additional PPE such as respirators may be necessary depending upon material and concentration. (Note: You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator). If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and call 911 to contact EH&S. Use appropriate tools and place in an appropriate waste disposal container and dispose of as hazardous waste (see above WASTE DISPOSAL PROCEDURES). As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on Paraformaldehyde can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | Keep container tightly closed and stored in a cool, dry place away from sunlight, heat, sparks, flame and other ignition sources. Store segregated from incompatible chemicals (below). Keep away from incompatibles such as oxidizing agents, reducing agents, metals, and acids. Paraformaldehyde decomposes slowly in WATER to form toxic and flammable formaldehyde gas. Training should include the following: Review of this SOP, current SDS, DOSH Lab Standard, DOSH Formaldehyde Standard, WSU Chemical Hygiene Plan, and special training provided by the department/supervisor. |

Certification of Hazard Assessment

Is this document a certification of Hazard Assessment for the processes identified within?                            			Yes					No

If yes, provide the name of the person certifying the Hazard Assessment and the date it was performed:

____________________________________________________________________________________

Name									Date

The location of the Hazard Assessment is indicated in the document preceding this form.

Certificate of Employee Training

Name of person providing training for employees working with this process:

___________________________________________________________________________________

The following employees have been trained in when, where and how to use selected PPE, the maintenance, limitations and disposal of the PPE selected, and have demonstrated the correct use of the PPE selected on the reverse of this certification.

Name								Date Trained

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________