# STANDARD OPERATING PROCEDURES FOR HAZARDOUS AND PARTICULARLY HAZARDOUS CHEMICALS

For

## Xylenes

| 1. PROCEDURE / PROCESS | Xylenes are used in Building, Room. Insert procedure here: |
| --- | --- |
| 2. CHEMICAL NAME(S) and associated PHYSICAL and HEALTH HAZARDS | Xylenes – CAS# 1330-20-7; also known as Xylenes (o-, m-, p- isomers); Dimethylbenzene; xylol; methyltoluene - Flammable Liquid and Vapor. - Harmful in contact with skin or if inhaled. - Causes skin and eye irritation. - The substance, which can be absorbed through the skin, may be toxic to blood, kidneys, liver, mucous membranes, bone marrow, central nervous system (CNS). Repeated or prolonged exposure to the substance can produce target organs damage. - Xylenes may contain up to 30% ethylbenzene, which is classified by the IARC as Group 2B: Possibly Carcinogenic. Signal Word: WARNING Exposure Limits: DOSH: TWA: 100 ppm; STEL: 150 ppm NIOSH: TWA: 100 ppm; STEL: 150 ppm ACGIH: TWA: 100 ppm; STEL: 150 ppm Toxicological Data: ORAL (LD50): 4,300 mg/kg [Rat]; 2,119 mg/kg [Mouse] DERMAL (LD50): 1,700 mg/kg [Rabbit] VAPOR (LC50): 49,000 mg/m 4 hours [Rat]; 440 ppm 24 hours [Mouse] *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. NAME OF TRAINER / RESOURCE PERSON | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for Xylenes is located in the Laboratory Safety Manual located in Building, Room. Labeling: Containers shall either have original warning label affixed or a label identifying the contents and hazards. |
| 5. PROTECTIVE EQUIPMENT | Wear chemical safety goggles, nitrile gloves and a fully-buttoned lab coat. If gloves are contacted, replace them immediately. For more than incidental splash contact, wear Viton or PVA gloves. Wash hands after removing gloves. Always work within a properly functioning certified laboratory fume hood. |
| - WASTE DISPOSAL PROCEDURES | Waste Xylenes must be collected in its pure form and solutions. It should be collected in a sealable, airtight, compatible waste container. The container should be stored away from incompatible materials such as oxidizing agents and acids. A completed Dangerous Waste label should be attached when waste is first added to the container. When container is full or no longer being used, complete a Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator at Building, Room, Phone Number. |
| 7. DESIGNATED AREA INFORMATION | The xylenes are stored and dispensed in Building, Room. Always work in a properly functioning, certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in Laboratories Chemical Hygiene Plan. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes. Consult a physician. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and immediately seek medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Spill: If a large amount of Xylenes is spilled outside the fume hood, immediately evacuate and secure area and call 911 to contact EH&S. Small Spill: If a small amount of Xylenes is spilled (it can be cleaned up in 10 minutes or less) and you have been appropriately trained to clean it up, you may do so. Trained personnel should wear at the minimum nitrile rubber, Viton or PVA gloves, chemical safety goggles, and a fully-buttoned lab coat. Additional PPE such as respirators may be necessary depending upon material and concentration. (Note: You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator). If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and call 911 to contact EH&S. Absorb with an inert dry material and place material in an appropriate waste disposal container (resealable bag, etc.) and dispose of as hazardous waste (see above WASTE DISPOSAL PROCEDURES). As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on Xylenes can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | Store in a tightly closed container in a secured, cool, dry and well-ventilated area. Store segregated from incompatible chemicals (below).Store as a flammable material. Store away from direct sunlight, heat, sparks, flame or any other source of ignition. Keep away from incompatibles such as oxidizing agents and acids. |

Certification of Hazard Assessment

Is this document a certification of Hazard Assessment for the processes identified within?                            			Yes					No

If yes, provide the name of the person certifying the Hazard Assessment and the date it was performed:

____________________________________________________________________________________

Name									Date

The location of the Hazard Assessment is indicated in the document preceding this form.

Certificate of Employee Training

Name of person providing training for employees working with this process:

___________________________________________________________________________________

The following employees have been trained in when, where and how to use selected PPE, the maintenance, limitations and disposal of the PPE selected, and have demonstrated the correct use of the PPE selected on the reverse of this certification.

Name								Date Trained

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________