# STANDARD OPERATING PROCEDURE

## Isoflurane

*Delete items in red after you have added all your lab-specific information. Modify general information as needed.

| 1. LAB INFORMATION | Building: Room(s): Department: Principal Investigator Name: Procedure: |
| --- | --- |
| 2. PHYSICAL & HEALTH HAZARDS | Isoflurane – CAS# 26675-46-7; also known as 2-chloro-2-(difluoromethoxy)-1,1,1-trifluoro-ethane; 1-chloro-2,2,2-trifluoroethyl difluoromethyl ether and forane is a halogenated ether used for inhalational anesthesia. It is a clear or colorless liquid that has a pungent or musty smell. Its use in human medicine is now starting to decline, being replaced with sevoflurane, desflurane and the intravenous anesthetic propofol. Isoflurane is still frequently used for veterinary anesthesia. - Anesthetic agent and central nervous system depressant. - May cause drowsiness or dizziness. - May cause eye, nose, throat and respiratory system irritation. - May cause damage to organs (Cardio-vascular system, central nervous system) through prolonged or repeated exposure if inhaled. - Epidemiological studies have indicated human over exposure to isoflurane has been associated with reproductive effects, congenital abnormalities, and adverse effects on the liver and kidneys. - Vapor is heavier than air and may accumulate in low spaces. GHS hazard pictograms Signal Word: WARNING Exposure Limits: ACGIH: 50 ppm (8-Hour TWA) Toxicological Data: ORAL (LD50): 4,770 mg/kg [Rat]. ORAL (LDLO): 1,657 mg/kg [Human-male]. INHALATION (LC50): 15,300 ppm 3 hours [Rat]; 16,800 ppm [Mouse]. *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. TRAINER / RESOURCE PERSONNEL | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for Isoflurane must be available at all times to all personnel working in the laboratory. The SDS used in this laboratory are located in Building, Room and/or Electronic Location. Labeling: Isoflurane containers shall either have original manufacturer’s label affixed or a self-made workplace label identifying the contents and hazards of the chemical at the minimum. Chemical abbreviations and formulae should be avoided unless a legend is present in the lab. For information on workplace labels, see https://ehs.wsu.edu/ohs-chemhazardcommunication/ohs-workplacelabels/. |
| 5. PROTECTIVE EQUIPMENT | Wear nitrile gloves, chemical splash goggles, and a fully buttoned lab coat at the minimum. ANSI approved safety glasses may be appropriate when properly working behind a fume hood’s protective sash. - Note: Always consult with your glove manufacturer’s glove compatibility chart to ensure selected gloves are compatible with specific chemical being used. Full length pants and close-toed shoes are required to be worn at all times. Insert lab specific information on personal protective equipment required for use. Always work within a properly functioning, face velocity certified laboratory chemical fume hood or under other local exhaust ventilation (ex: snorkel vent) as appropriate following hazard assessment. Insert lab specific information on ventilation controls and equipment used to control exposure. |
| - WASTE DISPOSAL PROCEDURES | Waste Isoflurane must be managed as Dangerous Waste. Collect solution in a compatible container (e.g. a polyethylene container provided by EH&S). Specialized waste containers and/or lids may be required for comingled wastes where reactions may generate heat or pressure. Waste containers should be stored away from incompatible materials (reference Section 10 of the SDS for incompatible materials). A completed Dangerous Waste label must be attached to dangerous waste containers before waste is added. When the container is full or no longer being used, submit an online Chemical Collection Request Form, and deliver to the Waste Accumulation Area Operator (if applicable) at Building, Room, Phone Number. Dangerous waste information and instructions can be found at the following link: https://ehs.wsu.edu/Chemical-Waste/ Do not dispose of Isoflurane waste down the drain or in the regular waste stream. For assistance evaluating dangerous waste characteristics, contact EH&S https://ehs.wsu.edu/contact-information/. Insert lab specific information on Isoflurane waste collection procedures, location, storage and handling. |
| 7. DESIGNATED AREA INFORMATION | The Isoflurane is stored and dispensed in Building, Room. Confine all work with Isoflurane to a properly functioning, face velocity certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in Laboratory’s Chemical Hygiene Plan (CHP). Insert lab specific information on Isoflurane storage and use locations. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and seek medical attention. In case of skin contact, flush skin with copious amounts of water for 15 minutes and seek medical attention. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Seek medical attention. In case of inhalation, move person to fresh air and seek medical attention. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Release: If a significant amount of Isoflurane is spilled outside the fume hood, immediately evacuate, secure area, notify others in the vicinity, and call 911 if there is an immediate threat of fire or the spill is uncontained. If personnel are exposed and need medical attention, call 911. If the spill is contained, secure the area, evacuate and call EH&S at 509-335-9000. Small Release: If a small amount of Isoflurane is spilled (it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel shall wear nitrile gloves, chemical splash goggles, and a fully-buttoned lab coat. Absorb with an inert dry material and place material in an appropriate waste disposal container (resealable bag, etc.) and dispose of as hazardous waste (see above WASTE DISPOSAL PROCEDURES). Additional PPE such as respirators may be necessary depending upon material and concentration. You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator. If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and follow the large release instructions above. As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on Isoflurane can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). Insert lab specific information on Isoflurane spill cleanup procedures and applicable location specific emergency procedures. |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | - Store Isoflurane containers upright in a designated, labeled area such as a chemical storage cabinet. Secondary containment such as a Nalgene/polypropylene tub is recommended. Isoflurane containers should always be stored below eye level. Storage area should be a secured, cool and well-ventilated area away from direct sunlight, heat, sparks, flame, or other sources of ignition. - Store Isoflurane in a designated, labeled, secure storage area away from other types of chemicals. Avoid incompatible chemicals such as strong oxidizers (refer to Section 10 of SDS). If storage space is limited, use secondary containment such as a Nalgene/polypropylene tubs. - Store in tightly closed original container until ready for use. Immediately close all containers after use. Containers should be in good condition and compatible with material. - Transport Isoflurane between locations in secondary containment, such as polyethylene or other non-reactive, non-breakable bottle carrier. - Purchase and store smallest quantities and lowest concentration needed for experimentation and what can be stored safely in the laboratory. Insert additional lab specific information on Isoflurane controls. |

- Certificate of Employee Training

The Principal Investigator must certify that they have provided training to lab personnel on the content of this SOP before work with Isoflurane is performed.

| PI Name | PI Signature | Date |
| --- | --- | --- |

The following individuals attest that they have received training from the Principal Investigator on the content of this SOP.

| Name | Signature | Date |
| --- | --- | --- |